# IOB.Connect™ P20 Gateway

## Installation Guide for Scotsman Prodigy Cuber Ice Machines

### Document Revision

Version: 2.0

Date:  6/23/2026

Prepared By: IOB, Inc.

# Overview

The IOB.Connect™ P20 Gateway provides real-time monitoring, remote diagnostics, two-way communication, and operational insights for Scotsman Prodigy Cuber Ice Machines [Scotsman].

This guide provides step-by-step instructions for installing and activating the IOB P20 Remote Monitoring Gateway [Gateway].

# Package Contents

Verify that the following components are included:

- P20 Gateway

- Cellular Antenna

- Serial Communication Cable

- Rubber Grommet

- DC Power Supply

Installer-Materials (not provided):

- Heavy-duty double-sided mounting tape or

- Self-tapping mounting screws

- Screwdrivers and basic hand tools

- Cable ties

# Safety Information

Before beginning installation:

- Disconnect power to the Scotsman ice machine.

- Follow all applicable electrical safety procedures.

- Do not install equipment while power is energized.

# Installation Requirements

Compatible Equipment:

- Scotsman Prodigy Cuber Series Ice Machines

Installation Requirements:

- Standard electrical outlet available for P20 power supply

- Adequate cellular signal strength

- Access to machine controller compartment

# Step 1 – Power Down Equipment

- Power OFF the ice machine.

- Disconnect power from the machine.

- Verify all power has been removed before proceeding.

# Step 2 – Remove Machine Panels

- Remove the front access panel.

- Remove the top panel (recommended), for easier accesses to Scotsman Controller.

- Retain all hardware for reinstallation.

Note:
Panel locations and fasteners may vary by model.

# Step 3 – Install Serial Communication Cable

Locate the cable access opening at the rear of the machine.

- On the back of the Scotsman, remove the bottom solid white grommet.

- Insert the supplied rubber grommet into the opening.

- Route the serial cable through the grommet. The BLACK end, with 6-pins.

- Open the Scotsman Controller compartment.

- Route the cable neatly to the controller board.

- Secure cable routing using cable ties.

# Step 4 – Connect to Scotsman Controller

- Locate the communication connector on the Scotsman controller board.

- Connect the BLACK 6-pin connector.

- Verify the connector is fully seated.

Important:
The connector is keyed and can only be installed in one orientation.

# Step 5 – Reinstall Machine Panels

- Close the controller compartment.

- Reinstall all removed panels.

- Verify that no cables are pinched or exposed.

# Step 6 – Record Gateway Information

Before mounting the gateway:

Record the following information:

Agent ID: ____________________

IMEI Number: ____________________

Take a photograph of the gateway label for future reference.

# Step 7 – Mount the Gateway

Select a mounting location:

- Protected from water exposure

- Away from moving components

- Accessible for service

- Provides adequate cellular signal

Mount using:

Option A:
Heavy-duty double-sided tape

Option B:
Self-tapping screws

Ensure mounting surfaces are clean and dry before installation.

# Step 8 – Connect Serial Cable to Gateway

- Connect the GREEN 4-pin connector to the gateway serial port.

- Ensure the connector is fully seated.

- Verify cable strain relief.

# Step 9 – Install Cellular Antenna

- Attach antenna to the gateway.

- Hand tighten only. Do NOT overtighten.

- Position antenna vertically.

For best cellular performance:

- Keep antenna unobstructed.

- Avoid metal obstructions where possible.

# Step 10 – Connect Power

- Insert the DC power connector into the gateway.

- Connect power supply to an AC outlet.

Gateway Startup:

- Three red LEDs indicate power is present.

- Cellular communication LED will flash during network registration.

- LED will become solid once connected.

Expected connection time:
1–3 minutes.

# Step 11 – Power Up Ice Machine

- Restore power to the ice machine.

- Verify normal machine operation.

# Step 12 – Activate Device

Navigate to:  https://lightning.hellothing.com

Activation Information Required:

- Agent ID

- IMEI Number

If you are a new user:

- Create an account.

- Register the gateway.

- Select a subscription plan (monthly / annual).

- Verify successful communication.

# Installation Verification Checklist

☐ Gateway mounted securely

☐ Antenna installed

☐ Serial cable connected

☐ Power connected

☐ LEDs indicate cellular connectivity

☐ Ice machine operational

☐ Device activated

☐ Machine Data visible on portal

# Troubleshooting

## No Power LEDs

Possible Causes:

- Power supply disconnected

- Outlet not energized

- Defective power adapter

Actions:

- Verify outlet power

- Verify DC connection

- Replace power supply if necessary

# Gateway Not Connecting to Cellular Network

Possible Causes:

- Poor signal strength

- Antenna disconnected

- SIM provisioning issue

Actions:

- Verify antenna connection

- Relocate antenna if necessary, (Optional Booster Antenna)

- Contact IOB Support

# No Machine Data Visible

Possible Causes:

- Serial cable disconnected

- Incorrect controller connection

- Device not activated

Actions:

- Verify all cable connections

- Verify portal activation

- Restart gateway

# Technical Support

For assistance, contact:

Email: Support@IOB.Inc

Please include:

- Agent ID

- IMEI Number

- Ice Machine Model Number

- Description of Issue

- Photos of Installation (if applicable)

# Installation Record

Customer: _____________________

Location: _____________________

Machine Model: _____________________

Gateway Agent ID: _____________________

Gateway IMEI: _____________________

Installed By: _____________________

Installation Date: _____________________